## Title and Citation

Radar for Europa Assessment and Sounding: Ocean to Near-Surface (REASON)
 Authors: D.D. Blankenship et al.
 Published in: Space Science Reviews (2024) 220:51
 DOI: 10.1007/s11214-024-01072-3
 Pages: 91

## 1. Scientific Background and Purpose

Europa is believed to host a global subsurface ocean under an icy shell. The Europa Clipper mission was developed to assess the moon’s potential for habitability by investigating key processes occurring between the exosphere, surface, ice shell, and ocean. The REASON radar is central to this task. Its goal is to “sound” Europa from its exosphere to its subsurface ocean, enabling a three-dimensional understanding of the moon’s structure and habitability-related processes.

### Key Hypotheses to Be Tested:

- The ice shell contains pockets of liquid water and brines.

- The shell overlies a global ocean that is decoupled from the rocky mantle and subject to tidal flexing.

- Material exchange occurs between the surface and interior, supporting potential habitability.

## 2. Radar Heritage and Instrument Design Philosophy

### 2.1 Technical Predecessors

REASON’s design builds on radar heritage from:

- Earth’s ice sheets (e.g., Antarctica and Greenland)

- Lunar and Martian radar sounders (e.g., SHARAD and MARSIS)
 These analogs helped define frequency bands, penetration depth, and resolution trade-offs.

### 2.2 Instrument Overview

REASON is a dual-frequency radar system, with:

- HF channel (9 MHz, 1 MHz bandwidth) for deep sounding up to 30 km depth.

- VHF channel (60 MHz, 10 MHz bandwidth) for high-resolution imaging of upper 3 km.

The two bands are designed to operate simultaneously, offering both deep penetration (HF) and high vertical resolution (VHF). The HF channel is more resilient to surface roughness, while the VHF is less affected by Jovian radio noise.

## 3. Science Objectives and Measurement Techniques

### 3.1 Measurement Techniques (Six Core Methods)

- Altimetry – Measures surface topography to determine tidal flexing and support hydrostatic equilibrium calculations.

- Reflectometry – Characterizes surface and near-surface layers, including brines, salt content, porosity, and geological roughness.

- Sounding – Penetrates the ice shell to reveal internal layering and search for the ice–ocean interface.

- Interferometry – Uses dual VHF channels to distinguish off-nadir surface clutter from nadir subsurface echoes.

- Plasma Characterization – Calculates Total Electron Content (TEC) to correct for ionospheric effects and detect active plumes.

- Ranging – Measures time-variable distances at crossover points to determine tidal deformation and constrain the Love number h₂.

## 4. Guiding and Extended Science Datasets

### 4.1 Four Guiding Science Datasets

- Near-Surface Properties: Regolith cohesiveness, surface roughness, subsurface porosity, potential plume fallout.

- Shallow Subsurface Exchange: Locations and properties of brine lenses or melt layers in the upper 3 km.

- Full Depth Subsurface Exchange: Vertical structure, thermophysical gradients, convective cells, and conductivity variations.

- Ice–Ocean Interface: Search for a reflective boundary and assess thickness of the shell.

### 4.2 Extended Science Datasets

- Tidal deformation measurements (Love number h₂)

- Thermal anomaly detection

- Landing site reconnaissance

- Subsurface geologic structures

- Plume-associated anomalies

- Global ionospheric mapping

- Clutter characterization for radargram interpretation

## 5. Radar Subsystems and Hardware

### 5.1 Hardware Subsystems

- Digital Electronics Subsystem (DSES): Controls radar signal generation, data processing, and packetization.

- Radio Frequency Electronics Subsystem (RFES): Contains high-power amplifiers and loopback calibration.

- Antenna Subsystem (AS): Includes 2 HF and 4 VHF antennas mounted on solar arrays.

- Power Converter Units (DSEPCU): Supplies regulated power to each subsystem.

### 5.2 Environmental Design

- Designed for the harsh Jovian radiation environment.

- Uses radiation-hardened materials and mitigates internal electrostatic discharge (IESD).

- Underwent extensive thermal vacuum, EMI/EMC, and radiation testing.

## 6. Calibration, Validation, and Measurement Quality

REASON uses a combination of pre-launch testing, in-flight calibration, and science validation through modeling:

- Ensemble of Point Models (EPMs): Simulate plausible Europa surface and subsurface conditions to test REASON’s signal detectability and resolution across variable dielectric properties.

- Key Radar Parameters (KRPs): Define requirements such as clutter discrimination, radiometric stability, vertical resolution, blind zone minimization, and side-lobe suppression.

## 7. Instrument Operations and Data Processing

### 7.1 Operations Systems

- Experiment Planning System (EPS): Coordinates science observation plans per flyby.

- Instrument Operations System (IOS): Prepares commands and monitors instrument health.

- Instrument Science Data System (iSDS): Processes raw data into calibrated science products.

- Instrument Science Team (iST): Manages science interpretation and algorithm development.

### 7.2 Concept of Operations

- Radar operations occur during ∼16-minute flyby windows.

- HF is only used when Jovian radio emissions are low.

- VHF is used more frequently and for interferometry.

- Radar parameters (gain, pulse width, receive delay) are reconfigured dwell-by-dwell.

## 8. Data Products and Archiving

REASON will generate several layers of science products:

- Raw Products (RAW): Fast-time radar returns and telemetry

- Partially Processed Data Products (PPDPs): Aligned radargrams and preliminary metadata

- Calibrated Data Products: Corrected for ionospheric effects and compressed

- Derived Products: Elevation models, TEC maps, thermal gradient profiles

- Special and Browse Products: Interpreted features and science summaries

All data will be made public via NASA’s Planetary Data System (PDS).

## 9. Complementary Science and Passive Sounding

REASON can perform passive radar sounding using Jovian decametric radio emissions. These emissions can be reflected from Europa’s surface and subsurface and used for analysis without transmitting any radar pulses—useful in high-noise regions or for conserving power.

It also complements the JUICE RIME radar. The two spacecraft may perform simultaneous observations, offering a form of bistatic sounding for plasma and subsurface diagnostics.

## 10. Summary and Scientific Impact

REASON is the most advanced radar ever flown to the outer solar system. It is purpose-built to:

- Penetrate Europa’s ice shell to depths of up to 30 kilometers.

- Search for subsurface lakes, brines, and the global ocean.

- Detect exchange processes between the surface and ocean.

- Reveal the internal structure, thermal gradients, and deformation behavior of the ice shell.

- Contribute crucial context to future exploration, including lander site selection.

In tandem with other Europa Clipper instruments, REASON will provide the definitive geophysical investigation of Europa’s habitability potential, setting the stage for the next generation of outer solar system missions.

# “Statistical analysis of 26 yr of observations of decametric radio emissions from Jupiter” by Marques et al., A&A 604, A17 (2017)

## Background

Jupiter is a strong and complex emitter of decametric radio emissions (DAM), ranging from 10 to 40 MHz. These emissions are generated by energetic electrons along Jupiter's magnetic field lines via the cyclotron-maser instability (CMI) mechanism—similar to Earth's auroral kilometric radiation.

DAM is modulated over a wide range of timescales:

- Milliseconds–seconds: S-bursts

- Seconds–minutes: Scintillations and modulation lanes

- Minutes–hours: Time-frequency (t–f) arcs

- ~10 hours: Planetary rotation

- Days–months: Influences from solar wind and interplanetary shocks

Two main types of emissions exist:

- Io-DAM: Controlled by the moon Io via electrodynamic interaction

- Non-Io DAM: Likely auroral in origin

These emissions are highly anisotropic (beamed), and their detection depends on Earth’s position relative to Jupiter.

## Observations

- Collected at Nançay Decameter Array (NDA) in France over 1990–2015.

- 8163 daily sessions (~8 hours each), totaling 54,578 hours of data.

- NDA: 144 log-spiral antennas, split into RH and LH circular polarizations.

- Spectral resolution: 75 kHz; time resolution: 1 s per spectrum per polarization.

- A catalog was built manually by visually inspecting dynamic spectra.

## Classification of Emissions

Using arc shape (vertex-late/early), polarization (RH/LH), and maximum frequency, emissions were classified into:

### Io-related (Io-DAM):

- Io-A (RH, vertex-late): Northern dusk limb, 38.6 MHz max freq.

- Io-A′ (RH, weak arc): Variant of Io-A, 33.4 MHz.

- Io-A″ (RH, vertex-late): Newly identified, overlaps with Io-C.

- Io-B (RH, vertex-early): Northern dawn limb, 38.9 MHz.

- Io-B′ (RH): New variant, ~34.5 MHz.

- Io-C (LH, vertex-late): Southern dusk limb, 30.4 MHz.

- Io-D (LH, vertex-early): Southern dawn limb, 29.4 MHz.

### Non-Io Emissions:

- non-Io-A (RH, vertex-late): 38.6 MHz.

- non-Io-B (RH, vertex-early): 36.1 MHz.

- non-Io-C (LH, vertex-late): 31.7 MHz.

- non-Io-D (LH, vertex-early): Newly identified, 28.6 MHz.

## Results & Analysis

### Occurrence & Visibility

- DAM detected 10.5% of observation time.

- Io emissions: 5.9%; non-Io: 4.6%.

- Io emissions dominate in intensity and duration.

- Io emissions occur in CML−ΦIo (Central Meridian Longitude vs. Io Phase) constrained zones.

- Non-Io emissions spread over CML, not modulated by Io phase.

### Hemispheric and Local Time Asymmetries

- Northern emissions are more frequent than southern ones.

- Io emissions: Higher max frequencies (up to 39 MHz in north).

- Non-Io emissions: Stronger on dusk side, likely due to solar wind interaction and auroral structures.

### Emission Properties

- Io-DAM has longer durations (~74 min avg) than non-Io-DAM (~41 min).

- Maximum frequencies:

  - Io-A, Io-B: ~39 MHz

  - Non-Io: ≤28 MHz (on average)

- Stokes V (circular polarization ratio): RH for north (negative), LH for south (positive), consistent with CMI theory.

### Temporal Trends

- Variations in emission types and strengths likely correlate with:

  - Solar wind fluctuations

  - Seasonal ionospheric changes

  - Io's volcanic activity

  - Magnetospheric dynamics

These trends are left for future studies.

## Significance of Io’s Longitude (ΛIo)

- Emissions clustered between ΛIo = 120°–300° (north) and 70°–360° (south).

- Maximum emission frequencies vary with ΛIo, supporting models of lead angle shifts due to Jupiter’s rotating magnetic field.

## Key Contributions

- Largest and most comprehensive DAM statistical study to date.

- First detection and categorization of Io-A″, Io-B′, and non-Io-D emission types.

- Provided time-frequency morphology, polarization, and frequency limits for each type.

- Data publicly available for further research.

## Instrumentation Note

- NDA’s sensitivity was limited; still, emissions were clearly detected and classified.

- RH and LH dynamic spectra allowed distinguishing between hemispheres.

## Conclusions

- Emissions are anisotropic and tightly linked to Jupiter’s rotation, magnetic field, and Io’s orbit.

- Io emissions are more consistent and intense; non-Io emissions are more variable and likely linked to auroral activity.

- The study provides a foundation for modeling the DAM emission mechanism and understanding Jupiter’s magnetosphere.